HERE IS A FULL CODE

// main.cpp - Merchant Ship IoT (Adafruit_SSD1306 version)

#include <Arduino.h>

#include <WiFi.h>

#include <WebServer.h>

#include <LittleFS.h>

#include <Wire.h>

#include <ArduinoJson.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <Adafruit_NeoPixel.h>

#include <DHT.h>

#include <TinyGPSPlus.h>

#include <ESP32Servo.h>

#include <MPU6050_tockn.h>

// ---------- BOOT BITMAPS (from Code1) ----------

const uint8_t shipBitmap[] PROGMEM = {

0x00,0x00,0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x10,0x00,0x00,

0x00,0x00,0x00,0x38,0x00,0x00,

0x00,0x00,0x00,0x38,0x00,0x00,

0x00,0x00,0x00,0x38,0x00,0x00,

0x00,0x00,0x00,0x38,0x00,0x00,

0x00,0x00,0x00,0x7C,0x00,0x00,

0x00,0x00,0x00,0xFE,0x00,0x00,

0x00,0x00,0x01,0xFF,0x00,0x00,

0x00,0x00,0x03,0xFF,0x80,0x00,

0x00,0x00,0x07,0xFF,0xC0,0x00,

0x00,0x00,0x0F,0xFF,0xE0,0x00,

0x00,0x00,0x1F,0xFF,0xF0,0x00,

0x00,0x00,0x3F,0xFF,0xF8,0x00,

0x7F,0xFF,0xFF,0xFF,0xFF,0xFE,

0x7F,0xFF,0xFF,0xFF,0xFF,0xFE,

0x3F,0xFF,0xFF,0xFF,0xFF,0xFC,

0x1F,0xFF,0xFF,0xFF,0xFF,0xF8,

0x0F,0xFF,0xFF,0xFF,0xFF,0xF0,

0x07,0xFF,0xFF,0xFF,0xFF,0xE0,

0x03,0xFF,0xFF,0xFF,0xFF,0xC0,

0x01,0xFF,0xFF,0xFF,0xFF,0x80,

0x00,0xFF,0xFF,0xFF,0xFF,0x00,

0x00,0x7F,0xFF,0xFF,0xFE,0x00

};

// ---------- CONFIG ----------

#define WIFI_SSID       "10xTC-AP2"

#define WIFI_PASS       "10xTechClub#"

#define SDA_PIN         21

#define SCL_PIN         22

#define TCA_ADDR        0x70

#define CH_OLED1        0

#define CH_OLED2        1

#define CH_MPU          2

#define DHT_ENGINE_PIN  25

#define DHT_OUTER_PIN   26

#define DHTTYPE         DHT11

#define MQ135_PIN       34

#define MOISTURE_PIN    35

#define US_TRIG_PIN     14

#define US_ECHO_PIN     27

// Servos pins (IMPORTANT)

#define SERVO_RADAR_PIN 18  // radar servo (0..180 sweep)

#define SERVO_CRANE1_PIN 19 // crane 1 - manual load/unload

#define SERVO_CRANE2_PIN 23 // crane 2 - manual load/unload

#define NEOPIXEL_PIN    4

#define NEOPIXEL_COUNT  8

#define GPS_RX          16

#define GPS_TX          17

#define GPS_BAUD        9600

#define OLED_ADDR       0x3C

#define BUZZER_PIN      33

#define GAS_ALERT_RAW       1800

#define OILTANK_LEVEL_LOW_PCT 15

#define PROX_ALERT_CM       30

#define RADAR_MIN_ANGLE     0

#define RADAR_MAX_ANGLE     180

#define RADAR_STEP          2

#define RADAR_SWEEP_MS      30

#define SENSOR_INTERVAL_MS  1000

#define OLED_INTERVAL_MS    800

#define GPS_INTERVAL_MS     250

#define STREAM_INTERVAL_MS  500

// ---------- GLOBALS ----------

WebServer server(80);

DHT dht_engine(DHT_ENGINE_PIN, DHTTYPE);

DHT dht_outer(DHT_OUTER_PIN, DHTTYPE);

Adafruit_NeoPixel pixels(NEOPIXEL_COUNT, NEOPIXEL_PIN, NEO_GRB + NEO_KHZ800);

HardwareSerial GPS_Serial(2);

TinyGPSPlus gps;

// Adafruit SSD1306 displays - same I2C address but we switch TCA channel before using init/operations

Adafruit_SSD1306 display1(128, 64, &Wire, -1);

Adafruit_SSD1306 display2(128, 64, &Wire, -1);

MPU6050 mpu(Wire);

Servo servoRadar, servoCrane1, servoCrane2;

// runtime state

float t_engine=0, h_engine=0, t_outer=0, h_outer=0;

uint16_t gas_raw=0, moisture_raw=0;

float oilLevelPct = -1;

float mpu_roll=0, mpu_pitch=0, mpu_yaw=0;

int sats=0;

double lat=0, lon=0;

String gps_time = "-";

volatile int radarAngle = 90; // start centered

int radarDir = +1;

float radarDistanceCm = -1;

int crane1Angle = 90, crane2Angle = 90; // current positions

int targetCrane1 = 90, targetCrane2 = 90; // desired positions (smooth move)

bool alert_gas=false, alert_leak=false, alert_prox=false;

unsigned long lastSensorMs=0, lastRadarMs=0, lastOledMs=0, lastGpsMs=0, lastStreamMs=0;

// SSE client

WiFiClient sseClient;

bool sseActive = false;

// ---------- HELPERS ----------

void tcaselect(uint8_t ch){

if (ch > 7) return;

Wire.beginTransmission(TCA_ADDR);

Wire.write(1 << ch);

Wire.endTransmission();

delayMicroseconds(100);

}

float readUltrasonicCm() {

digitalWrite(US_TRIG_PIN, LOW);

delayMicroseconds(3);

digitalWrite(US_TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(US_TRIG_PIN, LOW);

long dur = pulseIn(US_ECHO_PIN, HIGH, 30000UL);

if (dur == 0) return -1;

return dur / 58.0;

}

void setNeoPixelColor(uint8_t r, uint8_t g, uint8_t b) {

for (int i=0;i<NEOPIXEL_COUNT;i++){

pixels.setPixelColor(i, pixels.Color(r,g,b));

}

pixels.show();

}

String contentTypeFor(const String& path) {

if (path.endsWith(".html")) return "text/html";

if (path.endsWith(".css"))  return "text/css";

if (path.endsWith(".js"))   return "application/javascript";

if (path.endsWith(".json")) return "application/json";

if (path.endsWith(".png"))  return "image/png";

if (path.endsWith(".jpg"))  return "image/jpeg";

if (path.endsWith(".ico"))  return "image/x-icon";

return "text/plain";

}

bool serveFile(const String& path) {

String p = path;

if (!p.startsWith("/")) p = "/" + p;

if(!LittleFS.exists(p)) return false;

File file = LittleFS.open(p, "r");

if(!file) return false;

server.streamFile(file, contentTypeFor(p));

file.close();

return true;

}

// Helper: draw right aligned text using getTextBounds

void drawRightAligned(Adafruit_SSD1306 &d, int y, const String &s) {

int16_t x1,y1;

uint16_t w,h;

d.getTextBounds(s.c_str(), 0, y, &x1, &y1, &w, &h);

int16_t x = 127 - (int16_t)w;

if (x < 0) x = 0;

d.setCursor(x, y);

d.print(s);

}

// ---------- BOOT ANIMATION ----------

// Shows the ship bitmap + progress bar on both OLEDs (via tca select)

void showBootAnimation(){

for (int progress = 0; progress <= 100; progress += 10) {

int w = map(progress, 0, 100, 0, 100);

// OLED1

tcaselect(CH_OLED1);

display1.clearDisplay();

display1.setTextSize(1);

display1.setTextColor(SSD1306_WHITE);

display1.setCursor(20, 50);

display1.print("Loading Ship Data");

display1.drawRect(15, 35, 100, 10, SSD1306_WHITE);

if (w>0) display1.fillRect(15, 35, w, 10, SSD1306_WHITE);

// draw ship bitmap (bitmap width 48, height 24)

display1.drawBitmap(40, 2, shipBitmap, 48, 24, SSD1306_WHITE);

display1.display();

// OLED2 (mirror)

tcaselect(CH_OLED2);

display2.clearDisplay();

display2.setTextSize(1);

display2.setTextColor(SSD1306_WHITE);

display2.setCursor(12, 50);

display2.print("Loading Ship Data");

display2.drawRect(15, 35, 100, 10, SSD1306_WHITE);

if (w>0) display2.fillRect(15, 35, w, 10, SSD1306_WHITE);

display2.drawBitmap(40, 2, shipBitmap, 48, 24, SSD1306_WHITE);

display2.display();

delay(300);

}

delay(600);

}

// ---------- Display updates ----------

void updateDisplays() {

// OLED1: Engine Room + Oil + Alerts

tcaselect(CH_OLED1);

display1.clearDisplay();

display1.setTextSize(1);

display1.setTextColor(SSD1306_WHITE);

display1.setCursor(0, 0);

display1.print("Engine Room");

display1.setCursor(0, 12);

display1.print("T: ");

display1.print(t_engine,1);

display1.print("C  H: ");

display1.print(h_engine,0);

display1.print("%");

display1.setCursor(0, 24);

display1.print("Gas: ");

display1.print(gas_raw);

display1.setCursor(0, 36);

if (oilLevelPct < 0) display1.print("Oil Tank: No fuel");

else {

display1.print("Oil Level: ");

display1.print((int)oilLevelPct);

display1.print("%");

}

String al = String(alert_gas?"GAS ":"") + String(alert_leak?"OILLOW ":"") + String(alert_prox?"PROX ":"");

if (al.length() == 0) al = "None";

display1.setCursor(0, 48);

display1.print("ALERTS: ");

display1.print(al);

// right column: Radar

display1.setTextSize(1);

drawRightAligned(display1, 0, String("Radar"));

drawRightAligned(display1, 12, String("Ang: ") + String(radarAngle));

drawRightAligned(display1, 24, String("Dist: ") + (radarDistanceCm < 0 ? String("-") : String((int)radarDistanceCm) + "cm"));

display1.display();

// OLED2: GPS + Outer

tcaselect(CH_OLED2);

display2.clearDisplay();

display2.setTextSize(1);

display2.setTextColor(SSD1306_WHITE);

display2.setCursor(0, 0);

display2.print("GPS: ");

display2.print(lat, 4);

display2.print(",");

display2.print(lon, 4);

display2.setCursor(0, 10);

display2.print("Sats:");

display2.print(sats);

display2.print(" ");

display2.print(gps_time);

display2.setCursor(0, 20);

display2.print("Out:");

display2.print(t_outer,1);

display2.print("C ");

display2.print(h_outer,0);

display2.print("%");

String sys = (alert_gas || alert_leak || alert_prox) ? "ALERT!" : "OK";

drawRightAligned(display2, 20, sys);

display2.display();

}

// ---------- sensor & logic ----------

void computeOilLevel() {

if (moisture_raw > 4000) {

oilLevelPct = -1;

} else {

float inv = (4095.0 - (float)moisture_raw);

oilLevelPct = (inv / 4095.0) * 100.0;

if (oilLevelPct < 0) oilLevelPct = 0;

if (oilLevelPct > 100) oilLevelPct = 100;

}

if (oilLevelPct >= 0) alert_leak = (oilLevelPct <= OILTANK_LEVEL_LOW_PCT);

else alert_leak = false;

}

void updateNeoAndBuzzer() {

if (alert_gas || alert_leak || alert_prox) {

setNeoPixelColor(255,0,0);

digitalWrite(BUZZER_PIN, HIGH);

} else {

setNeoPixelColor(0,180,0);

digitalWrite(BUZZER_PIN, LOW);

}

}

void readSensors() {

float te = dht_engine.readTemperature();

float he = dht_engine.readHumidity();

float to = dht_outer.readTemperature();

float ho = dht_outer.readHumidity();

if (!isnan(te)) t_engine = te;

if (!isnan(he)) h_engine = he;

if (!isnan(to)) t_outer = to;

if (!isnan(ho)) h_outer = ho;

gas_raw = analogRead(MQ135_PIN);

moisture_raw = analogRead(MOISTURE_PIN);

computeOilLevel();

tcaselect(CH_MPU);

mpu.update();

mpu_roll  = mpu.getAngleX();

mpu_pitch = mpu.getAngleY();

mpu_yaw   = mpu.getAngleZ();

alert_prox = (radarDistanceCm > 0 && radarDistanceCm <= PROX_ALERT_CM);

alert_gas = (gas_raw >= GAS_ALERT_RAW);

updateNeoAndBuzzer();

}

// Take multiple ultrasonic samples and return median

float readUltrasonicMedianCm(uint8_t samples = 5) {

float vals[9]; // max 9 samples

samples = constrain(samples, 1, 9);

for (uint8_t i = 0; i < samples; i++) {

vals[i] = readUltrasonicCm();

delayMicroseconds(200); // small gap

}

// sort

for (uint8_t i = 0; i < samples - 1; i++) {

for (uint8_t j = i + 1; j < samples; j++) {

if (vals[j] < vals[i]) {

float t = vals[i]; vals[i] = vals[j]; vals[j] = t;

}

}

}

return vals[samples / 2]; // median

}

// Optional moving average buffer

#define RADAR_FILTER_LEN 4

float radarBuf[RADAR_FILTER_LEN];

uint8_t radarBufIdx = 0;

bool radarBufFilled = false;

float filterRadar(float v) {

if (v < 0) return -1; // invalid

radarBuf[radarBufIdx++] = v;

if (radarBufIdx >= RADAR_FILTER_LEN) { radarBufIdx = 0; radarBufFilled = true; }

int count = radarBufFilled ? RADAR_FILTER_LEN : radarBufIdx;

float sum = 0; for (int i = 0; i < count; i++) sum += radarBuf[i];

return sum / count;

}

// ---------- radar sweep ----------

void sweepRadar() {

unsigned long now = millis();

if (now - lastRadarMs >= RADAR_SWEEP_MS) {

lastRadarMs = now;

// move angle

radarAngle += radarDir * RADAR_STEP;

if (radarAngle >= RADAR_MAX_ANGLE) { radarAngle = RADAR_MAX_ANGLE; radarDir = -1; }

if (radarAngle <= RADAR_MIN_ANGLE) { radarAngle = RADAR_MIN_ANGLE; radarDir = +1; }

servoRadar.write(radarAngle);

delay(8); // allow servo to settle slightly

// read distance with median + filter

float raw = readUltrasonicMedianCm(3);

if (raw <= 0 || raw > 350) radarDistanceCm = -1; // out of range

else radarDistanceCm = filterRadar(raw);

}

}

// ---------- GPS ----------

void readGPS() {

while (GPS_Serial.available()) gps.encode(GPS_Serial.read());

if (gps.location.isValid()) { lat = gps.location.lat(); lon = gps.location.lng(); }

if (gps.satellites.isValid()) sats = gps.satellites.value();

if (gps.time.isValid()) {

char buf[16];

snprintf(buf, sizeof(buf), "%02d:%02d:%02d",

gps.time.hour(), gps.time.minute(), gps.time.second());

gps_time = String(buf);

}

}

// ---------- smooth servo for cranes ----------

void smoothMoveServo(Servo &servo, int &currentAngle, int target, int step=3) {

if (currentAngle == target) return;

if (abs(currentAngle - target) <= step) {

currentAngle = target;

} else if (currentAngle < target) currentAngle += step;

else currentAngle -= step;

servo.write(currentAngle);

}

// ---------- SSE helpers ----------

void sseInitClient() {

// nothing here — we accept client in handler and store sseClient

}

void sseSend(const String &payload) {

if (!sseActive) return;

if (!sseClient.connected()) {

sseActive = false;

sseClient.stop();

Serial.println("[SSE] Client disconnected");

return;

}

sseClient.print("data: ");

sseClient.print(payload);

sseClient.print("\n\n");

sseClient.flush();

}

// ---------- HTTP handlers ----------

void handleRoot() { if(!serveFile("/index.html")) server.send(404,"text/plain","index.html not found"); }

void handleStatic() {

String path = server.uri();

if (!serveFile(path)) server.send(404,"text/plain","not found");

}

void handleFavicon(){ if(!serveFile("/favicon.ico")) server.send(204,"text/plain"," "); }

void handleData() {

StaticJsonDocument<2048> doc;

JsonObject eng = doc.createNestedObject("engine_room");

eng["temp_c"] = t_engine;

eng["hum_pct"] = h_engine;

eng["gas_raw"] = gas_raw;

eng["oil_level_pct"] = (oilLevelPct < 0 ? -1 : (int)oilLevelPct);

JsonObject outer = doc.createNestedObject("outer");

outer["temp_c"] = t_outer;

outer["hum_pct"] = h_outer;

JsonObject gpsj = doc.createNestedObject("gps");

gpsj["lat"] = lat; gpsj["lon"] = lon; gpsj["sats"] = sats; gpsj["time"] = gps_time;

JsonObject radar = doc.createNestedObject("radar");

radar["angle"] = radarAngle;

radar["distance_cm"] = (radarDistanceCm < 0 ? -1 : (int)radarDistanceCm);

JsonObject crane = doc.createNestedObject("crane");

crane["s1"] = crane1Angle; crane["s2"] = crane2Angle;

JsonObject alerts = doc.createNestedObject("alerts");

alerts["gas"] = alert_gas; alerts["oilLow"] = alert_leak; alerts["proximity"] = alert_prox; alerts["any"] = (alert_gas||alert_leak||alert_prox);

String out; serializeJson(doc, out);

server.send(200, "application/json", out);

}

void logAndJsonResponse(const char* tag, int servoId, int angle) {

Serial.printf("[HTTP] %s requested: servo=%d angle=%d\n", tag, servoId, angle);

StaticJsonDocument<128> doc;

doc["ok"] = true;

doc["servo"] = servoId;

doc["angle"] = angle;

String out; serializeJson(doc, out);

server.send(200, "application/json", out);

}

void handleControl() {

// GET style: /control?servo=1&angle=90

if (server.method() == HTTP_GET) {

if (!server.hasArg("servo") || !server.hasArg("angle")) {

server.send(400,"application/json","{\"ok\":false,\"error\":\"missing servo or angle\"}");

return;

}

int s = server.arg("servo").toInt();

int a = constrain(server.arg("angle").toInt(), 0, 180);

if (s==0) {

// radar — write immediate

radarAngle = a; servoRadar.write(radarAngle);

Serial.printf("[CONTROL] radar set immediate to %d\n", radarAngle);

logAndJsonResponse("control", s, a);

return;

} else if (s==1) { targetCrane1 = a; Serial.printf("[CONTROL] set target crane1=%d\n", a); logAndJsonResponse("control", s, a); return;}

else if (s==2) { targetCrane2 = a; Serial.printf("[CONTROL] set target crane2=%d\n", a); logAndJsonResponse("control", s, a); return;}

else { server.send(400,"application/json","{\"ok\":false,\"error\":\"servo must be 0/1/2\"}"); return; }

}

// POST JSON body { "servo":1,"angle":90 }

if (server.method() == HTTP_POST) {

String body = server.arg("plain");

StaticJsonDocument<256> rdoc;

DeserializationError err = deserializeJson(rdoc, body);

if (err) { server.send(400,"application/json","{\"ok\":false,\"error\":\"invalid json\"}"); return; }

int s = rdoc["servo"] | -1; int a = rdoc["angle"] | -1;

if (s<0 || a<0) { server.send(400,"application/json","{\"ok\":false,\"error\":\"missing fields\"}"); return; }

a = constrain(a,0,180);

if (s==0) { radarAngle = a; servoRadar.write(radarAngle); Serial.printf("[CONTROL POST] radar immediate %d\n", a); logAndJsonResponse("control_post", s, a); return; }

if (s==1) { targetCrane1 = a; Serial.printf("[CONTROL POST] crane1 target %d\n", a); logAndJsonResponse("control_post", s, a); return; }

if (s==2) { targetCrane2 = a; Serial.printf("[CONTROL POST] crane2 target %d\n", a); logAndJsonResponse("control_post", s, a); return; }

server.send(400,"application/json","{\"ok\":false,\"error\":\"invalid servo id\"}");

return;

}

server.send(405,"application/json","{\"ok\":false,\"error\":\"method not allowed\"}");

}

void handleServoRadar() {

if (!server.hasArg("angle")) { server.send(400,"application/json","{\"ok\":false,\"error\":\"missing angle\"}"); return; }

int a = constrain(server.arg("angle").toInt(), 0, 180);

radarAngle = a; servoRadar.write(radarAngle);

Serial.printf("[HTTP] /servoRadar -> %d\n", a);

logAndJsonResponse("servoRadar", 0, a);

}

void handleServoCrane1() {

if (!server.hasArg("angle")) { server.send(400,"application/json","{\"ok\":false,\"error\":\"missing angle\"}"); return; }

int a = constrain(server.arg("angle").toInt(), 0, 180);

targetCrane1 = a;

Serial.printf("[HTTP] /servoCrane1 target -> %d\n", a);

logAndJsonResponse("servoCrane1", 1, a);

}

void handleServoCrane2() {

if (!server.hasArg("angle")) { server.send(400,"application/json","{\"ok\":false,\"error\":\"missing angle\"}"); return; }

int a = constrain(server.arg("angle").toInt(), 0, 180);

targetCrane2 = a;

Serial.printf("[HTTP] /servoCrane2 target -> %d\n", a);

logAndJsonResponse("servoCrane2", 2, a);

}

// compatibility /api/cmd (form style)

void handleApiCmd() {

Serial.println("[HTTP] /api/cmd called");

if (server.hasArg("servo") && server.hasArg("angle")) {

handleControl();

return;

}

// try parse body like servo=1&angle=90

if (server.method()==HTTP_POST) {

String b = server.arg("plain");

int sIndex = b.indexOf("servo=");

int aIndex = b.indexOf("angle=");

if (sIndex>=0 && aIndex>=0) {

int s = b.substring(sIndex+6).toInt();

int a = b.substring(aIndex+6).toInt();

if (s==0) { radarAngle=a; servoRadar.write(radarAngle); Serial.printf("[API CMD] radar immediate %d\n", a); }

else if (s==1) { targetCrane1=a; Serial.printf("[API CMD] crane1 target %d\n", a); }

else if (s==2) { targetCrane2=a; Serial.printf("[API CMD] crane2 target %d\n", a); }

StaticJsonDocument<128> doc; doc["ok"]=true; doc["servo"]=s; doc["angle"]=a; String out; serializeJson(doc,out); server.send(200,"application/json",out); return;

}

}

server.send(400,"application/json","{\"ok\":false,\"error\":\"bad request\"}");

}

// /stream SSE handshake

void handleStream() {

WiFiClient client = server.client();

if (!client) return;

// handshake headers

server.sendHeader("Cache-Control","no-cache");

server.sendHeader("Content-Type","text/event-stream");

server.sendHeader("Connection","keep-alive");

server.send(200, "text/event-stream", ""); // send HTTP headers

delay(10);

// store client for periodic pushes

sseClient = client;

sseActive = true;

Serial.println("[SSE] client connected");

}

// debug endpoint

void handleDebug() {

StaticJsonDocument<512> doc;

doc["radarAngle"]=radarAngle;

doc["radarDistanceCm"]=radarDistanceCm;

doc["crane1Angle"]=crane1Angle; doc["crane1Target"]=targetCrane1;

doc["crane2Angle"]=crane2Angle; doc["crane2Target"]=targetCrane2;

doc["alerts"]["gas"]=alert_gas; doc["alerts"]["oilLow"]=alert_leak; doc["alerts"]["prox"]=alert_prox;

String out; serializeJson(doc,out);

server.send(200,"application/json",out);

}

// ---------- SETUP ----------

void setup() {

Serial.begin(115200);

delay(100);

Serial.println("\n--- Merchant Ship IoT (Adafruit SSD1306 build) ---");

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

// I2C

Wire.begin(SDA_PIN, SCL_PIN);

Serial.println("I2C started");

// Displays: initialize each display while its TCA channel is selected

tcaselect(CH_OLED1);

if(!display1.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("SSD1306 OLED1 init failed");

} else {

display1.clearDisplay();

display1.display();

}

tcaselect(CH_OLED2);

if(!display2.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("SSD1306 OLED2 init failed");

} else {

display2.clearDisplay();

display2.display();

}

// show boot animation on both displays (uses tcaselect inside)

showBootAnimation();

// show brief boot text on OLED1

tcaselect(CH_OLED1);

display1.clearDisplay();

display1.setTextSize(2);

display1.setTextColor(SSD1306_WHITE);

display1.setCursor(0,0);

display1.print("Seabotics");

display1.setTextSize(1);

display1.setCursor(0,24);

display1.print("Booting...");

display1.display();

// DHT

dht_engine.begin(); dht_outer.begin();

Serial.println("DHTs init");

// ADC resolution

analogReadResolution(12);

// Ultrasonic pins

pinMode(US_TRIG_PIN, OUTPUT); pinMode(US_ECHO_PIN, INPUT);

// Servos

servoRadar.attach(SERVO_RADAR_PIN);

servoCrane1.attach(SERVO_CRANE1_PIN);

servoCrane2.attach(SERVO_CRANE2_PIN);

servoRadar.write(radarAngle);

servoCrane1.write(crane1Angle);

servoCrane2.write(crane2Angle);

Serial.printf("Servos attached (radar:%d, crane1:%d, crane2:%d)\n", SERVO_RADAR_PIN, SERVO_CRANE1_PIN, SERVO_CRANE2_PIN);

// NeoPixel

pixels.begin(); pixels.clear(); setNeoPixelColor(0,0,255);

// GPS

GPS_Serial.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX);

Serial.println("GPS started");

// MPU

tcaselect(CH_MPU);

mpu.begin(); delay(300);

Serial.println("Calibrating MPU... keep steady");

mpu.calcGyroOffsets(true);

Serial.println("MPU calibrated");

// LittleFS

if (!LittleFS.begin(true)) {

Serial.println("LittleFS mount failed (files may be missing)");

} else {

Serial.println("LittleFS mounted");

}

// WiFi

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.printf("Connecting to WiFi %s ...", WIFI_SSID);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 30) {

delay(500); Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.printf("\nWiFi connected: %s\n", WiFi.localIP().toString().c_str());

setNeoPixelColor(0,255,0);

} else {

Serial.println("\nWiFi failed");

setNeoPixelColor(255,100,0);

}

// HTTP routes

server.on("/", HTTP_GET, handleRoot);

server.on("/index.html", HTTP_GET, handleRoot);

server.on("/style.css", HTTP_GET, handleStatic);

server.on("/script.js", HTTP_GET, handleStatic);

server.on("/favicon.ico", HTTP_GET, handleFavicon);

server.on("/data", HTTP_GET, handleData);

server.on("/stream", HTTP_GET, handleStream);

server.on("/control", HTTP_ANY, handleControl);

server.on("/servoRadar", HTTP_GET, handleServoRadar);

server.on("/servoCrane1", HTTP_GET, handleServoCrane1);

server.on("/servoCrane2", HTTP_GET, handleServoCrane2);

server.on("/api/cmd", HTTP_ANY, handleApiCmd);

server.on("/debug", HTTP_GET, handleDebug);

server.onNotFound([](){ server.send(404,"text/plain","Not found"); });

server.begin();

Serial.println("HTTP server started");

// init timers

lastSensorMs = millis() - SENSOR_INTERVAL_MS;

lastRadarMs = millis() - RADAR_SWEEP_MS;

lastOledMs = millis() - OLED_INTERVAL_MS;

lastGpsMs = millis() - GPS_INTERVAL_MS;

lastStreamMs = millis() - STREAM_INTERVAL_MS;

}

// ---------- LOOP ----------

void loop() {

unsigned long now = millis();

server.handleClient();

// GPS

if (now - lastGpsMs >= GPS_INTERVAL_MS) {

lastGpsMs = now;

readGPS();

}

// Sensors

if (now - lastSensorMs >= SENSOR_INTERVAL_MS) {

lastSensorMs = now;

readSensors();

}

// Radar sweep (auto)

sweepRadar();

// Smooth crane moves

smoothMoveServo(servoCrane1, crane1Angle, targetCrane1, 3);

smoothMoveServo(servoCrane2, crane2Angle, targetCrane2, 3);

// OLED updates

if (now - lastOledMs >= OLED_INTERVAL_MS) {

lastOledMs = now;

updateDisplays();

}

// SSE periodic push (emit /data snapshot)

if (sseActive && sseClient.connected()) {

if (now - lastStreamMs >= STREAM_INTERVAL_MS) {

lastStreamMs = now;

StaticJsonDocument<1024> doc;

doc["t_engine"]=t_engine; doc["h_engine"]=h_engine; doc["gas_raw"]=gas_raw; doc["oil_level_pct"]=(oilLevelPct<0?-1:(int)oilLevelPct);

JsonObject gpsj = doc.createNestedObject("gps"); gpsj["lat"]=lat; gpsj["lon"]=lon; gpsj["sats"]=sats; gpsj["time"]=gps_time;

JsonObject radar = doc.createNestedObject("radar"); radar["angle"]=radarAngle; radar["distance_cm"]=(radarDistanceCm<0?-1:(int)radarDistanceCm);

JsonObject alerts = doc.createNestedObject("alerts"); alerts["gas"]=alert_gas; alerts["oilLow"]=alert_leak; alerts["prox"]=alert_prox; alerts["any"]=(alert_gas||alert_leak||alert_prox);

String out; serializeJson(doc,out);

sseSend(out);

}

} else {

// if previous client disconnected, reset flag

if (sseActive && !sseClient.connected()) {

sseActive = false;

sseClient.stop();

Serial.println("[SSE] client lost");

}

}

delay(1);

}

HERE IS A FULL BREAKDOWN OF THE CODE

1) Includes — what the code imports and why

#include <Arduino.h>

#include <WiFi.h>

#include <WebServer.h>

#include <LittleFS.h>

#include <Wire.h>

#include <ArduinoJson.h>

#include <Adafruit_NeoPixel.h>

#include <DHT.h>

#include <TinyGPSPlus.h>

#include <ESP32Servo.h>

#include <SSD1306Wire.h>

#include <MPU6050_tockn.h>

- Each #include brings a library.

  - Arduino.h — base Arduino functions (pinMode, digitalWrite, delay, etc.).

  - WiFi.h & WebServer.h — connect to Wi-Fi and create a small webserver.

  - LittleFS.h — filesystem on ESP32 flash for serving index.html, script.js, etc.

  - Wire.h — I2C communication (used with the TCA9548A multiplexer and MPU/OLEDs).

  - ArduinoJson.h — building and parsing JSON for APIs and SSE.

  - Adafruit_NeoPixel.h — control NeoPixel LED strip.

  - DHT.h — read DHT11 temp/humidity sensors.

  - TinyGPSPlus.h — parse NMEA sentences from GPS module.

  - ESP32Servo.h — servo control on ESP32 (timers/PWM wrapper).

  - SSD1306Wire.h — OLED driver for SSD1306 displays.

  - MPU6050_tockn.h — convenient wrapper to get angles from MPU6050.

2) Boot bitmap (static image stored in flash)

const uint8_t shipBitmap[] PROGMEM = { ... };

- PROGMEM stores the bitmap data in program memory (flash) instead of RAM because bitmaps are big.

- Used by the boot animation to draw a small ship on OLED.

3) Configuration macros — quick mapping between names and pins/values

#define WIFI_SSID "10xTC-AP2"

#define WIFI_PASS "10xTechClub#"

// ...

#define SERVO_RADAR_PIN 18

#define NEOPIXEL_PIN 4

#define GPS_RX 16

#define OLED_ADDR 0x3C

#define GAS_ALERT_RAW 1800

// timing constants

#define SENSOR_INTERVAL_MS 1000

#define OLED_INTERVAL_MS 800

- #define gives human-readable names to pins, thresholds, and intervals.

- Thresholds like GAS_ALERT_RAW and OILTANK_LEVEL_LOW_PCT decide when alerts trigger.

- Time constants control how often parts of the system update (sensors, OLED, GPS).

4) Global objects and variables

WebServer server(80);

DHT dht_engine(DHT_ENGINE_PIN, DHTTYPE);

Adafruit_NeoPixel pixels(NEOPIXEL_COUNT, NEOPIXEL_PIN, NEO_GRB + NEO_KHZ800);

HardwareSerial GPS_Serial(2);

TinyGPSPlus gps;

SSD1306Wire display1(OLED_ADDR, SDA_PIN, SCL_PIN);

MPU6050 mpu(Wire);

Servo servoRadar, servoCrane1, servoCrane2;

- Hardware/driver objects are created globally so all functions can use them.

- server(80) creates an HTTP server listening on port 80.

- GPS_Serial(2) uses UART2 (ESP32 has multiple hardware serial ports).

- display1 and display2 use same I2C address — multiplexer tcaselect() picks which one to talk to.

Runtime state variables:

float t_engine=0, h_engine=0, ...;

uint16_t gas_raw=0;

int radarAngle = 90;

int crane1Angle = 90, targetCrane1 = 90;

bool alert_gas=false, alert_leak=false, alert_prox=false;

unsigned long lastSensorMs=0, lastRadarMs=0, ...

WiFiClient sseClient;

bool sseActive = false;

- These store sensor readings, servo positions, alert flags, and timestamps for scheduling.

- radarAngle starts at 90 (center). targetCrane1 is used for smooth movement.

5) I2C multiplexer helper

void tcaselect(uint8_t ch){

if (ch > 7) return;

Wire.beginTransmission(TCA_ADDR);

Wire.write(1 << ch);

Wire.endTransmission();

delayMicroseconds(100);

}

- The multiplexer has channels 0–7. Writing 1<<ch to its address selects that channel.

- tcaselect(CH_OLED1) is called before talking to an OLED so the correct device responds.

6) Ultrasonic distance helper

float readUltrasonicCm() {

digitalWrite(US_TRIG_PIN, LOW);

delayMicroseconds(3);

digitalWrite(US_TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(US_TRIG_PIN, LOW);

long dur = pulseIn(US_ECHO_PIN, HIGH, 30000UL);

if (dur == 0) return -1;

return dur / 58.0;

}

- Triggers the HC-SR04: set TRIG HIGH briefly and measure the duration of ECHO using pulseIn.

- dur/58.0 is the common conversion to centimeters.

- If pulseIn times out it returns 0 → treat as -1 (invalid).

7) NeoPixel helper

void setNeoPixelColor(uint8_t r, uint8_t g, uint8_t b) {

for (int i=0;i<NEOPIXEL_COUNT;i++){

pixels.setPixelColor(i, pixels.Color(r,g,b));

}

pixels.show();

}

- Sets all NeoPixels to the same RGB color and refreshes the strip.

8) File serving helpers (LittleFS)

String contentTypeFor(const String& path) { ... }

bool serveFile(const String& path) {

if(!LittleFS.exists(p)) return false;

File file = LittleFS.open(p, "r");

server.streamFile(file, contentTypeFor(p));

file.close();

return true;

}

- contentTypeFor() picks a MIME type based on extension so browsers render files correctly.

- serveFile() reads files from LittleFS and streams them to the HTTP client. If file not found, returns false.

9) Boot animation

void showBootAnimation(){

for (int progress = 0; progress <= 100; progress += 10) {

tcaselect(CH_OLED1);

display1.clear();

display1.setFont(ArialMT_Plain_10);

display1.drawString(20, 50, "Loading Ship Data");

display1.drawRect(15, 35, 100, 10);

int w = map(progress, 0, 100, 0, 100);

if (w>0) display1.fillRect(15, 35, w, 10);

display1.drawXbm(50, 0, 24, 24, shipBitmap);

display1.display();

// mirror on OLED2

tcaselect(CH_OLED2);

...

delay(300);

}

}

- Loops progress 0→100, draws progress bar and the ship bitmap on both displays (select channel before each).

- drawXbm used to draw the shipBitmap.

- Visual feedback while system initializes.

10) JSON helper

void sendJson(WebServer &srv, const JsonDocument& doc) {

String out;

serializeJson(doc, out);

srv.send(200, "application/json", out);

}

- Serializes an ArduinoJson document into a string and sends it as application/json using the provided WebServer instance.

11) Display update function

void updateDisplays() {

tcaselect(CH_OLED1);

display1.clear();

...

display1.drawString(0, 0, "Engine Room");

display1.drawString(0, 12, "T: " + String(t_engine,1) + "C  H: " + String(h_engine,0) + "%");

...

display1.display();

tcaselect(CH_OLED2);

display2.clear();

display2.drawString(0, 0, "GPS: " + String(lat, 4) + "," + String(lon, 4));

...

display2.display();

}

- Writes a concise status to both OLEDs: engine room values, alerts, radar angle/distance, and GPS on the other display.

- String formatting uses String() with precision (e.g., String(t_engine,1) shows one decimal).

12) Oil level computation

void computeOilLevel() {

if (moisture_raw > 4000) {

oilLevelPct = -1;

} else {

float inv = (4095.0 - (float)moisture_raw);

oilLevelPct = (inv / 4095.0) * 100.0;

if (oilLevelPct >= 0) alert_leak = (oilLevelPct <= OILTANK_LEVEL_LOW_PCT);

}

}

- Converts analog reading (0–4095 at 12-bit ADC) to a percentage indicating how full the tank is.

- >4000 treated as invalid (sensor not in tank / disconnected) → -1.

- alert_leak becomes true if computed % ≤ threshold.

13) NeoPixel & buzzer update

void updateNeoAndBuzzer() {

if (alert_gas || alert_leak || alert_prox) {

setNeoPixelColor(255,0,0);

digitalWrite(BUZZER_PIN, HIGH);

} else {

setNeoPixelColor(0,180,0);

digitalWrite(BUZZER_PIN, LOW);

}

}

- Visual + audible alerts react to any alert flag.

- NeoPixel shows red if any alert; green if all ok.

14) Sensor reading routine

void readSensors() {

float te = dht_engine.readTemperature();

float he = dht_engine.readHumidity();

...

if (!isnan(te)) t_engine = te;

...

gas_raw = analogRead(MQ135_PIN);

moisture_raw = analogRead(MOISTURE_PIN);

computeOilLevel();

tcaselect(CH_MPU);

mpu.update();

mpu_roll  = mpu.getAngleX();

...

alert_prox = (radarDistanceCm > 0 && radarDistanceCm <= PROX_ALERT_CM);

alert_gas = (gas_raw >= GAS_ALERT_RAW);

updateNeoAndBuzzer();

}

- Reads DHTs and only updates stored values if reading is valid (!isnan()).

- Reads analog sensors (gas and moisture).

- Selects MPU channel and updates roll/pitch/yaw.

- Computes alert flags and updates outputs.

15) Ultrasonic median sampling and filter

float readUltrasonicMedianCm(uint8_t samples = 5) { ... }

#define RADAR_FILTER_LEN 4

float radarBuf[RADAR_FILTER_LEN];

float filterRadar(float v) { ... }

- readUltrasonicMedianCm samples the sensor multiple times and returns the median — robust against occasional spurious readings.

- filterRadar is a moving average over RADAR_FILTER_LEN samples to smooth the radar distance measurement.

16) Radar sweep logic

void sweepRadar() {

unsigned long now = millis();

if (now - lastRadarMs >= RADAR_SWEEP_MS) {

lastRadarMs = now;

radarAngle += radarDir * RADAR_STEP;

if (radarAngle >= RADAR_MAX_ANGLE) { radarAngle = RADAR_MAX_ANGLE; radarDir = -1; }

if (radarAngle <= RADAR_MIN_ANGLE) { radarAngle = RADAR_MIN_ANGLE; radarDir = +1; }

servoRadar.write(radarAngle);

delay(8);

float raw = readUltrasonicMedianCm(3);

if (raw <= 0 || raw > 350) radarDistanceCm = -1;

else radarDistanceCm = filterRadar(raw);

}

}

- Called frequently from loop(). When enough time passed, it:

  - Advances radarAngle by RADAR_STEP in current radarDir.

  - Reverses direction at endpoints (0 and 180).

  - Writes the angle to the servo.

  - Reads distance and filters it.

- delay(8) gives servo a small time to move before measuring.

17) GPS reading

void readGPS() {

while (GPS_Serial.available()) gps.encode(GPS_Serial.read());

if (gps.location.isValid()) { lat = gps.location.lat(); lon = gps.location.lng(); }

if (gps.satellites.isValid()) sats = gps.satellites.value();

if (gps.time.isValid()) { snprintf(buf,...); gps_time = String(buf); }

}

- Feeds incoming serial bytes to TinyGPSPlus which extracts location, time & satellites.

- Only updates values when isValid() is true.

18) Smooth servo motion for crane

void smoothMoveServo(Servo &servo, int &currentAngle, int target, int step=3) {

if (currentAngle == target) return;

if (abs(currentAngle - target) <= step) currentAngle = target;

else if (currentAngle < target) currentAngle += step;

else currentAngle -= step;

servo.write(currentAngle);

}

- Moves the servo a few degrees (default step=3) per call toward the target, producing smooth motion rather than instant jumps.

19) SSE (Server-Sent Events) helpers

WiFiClient sseClient;

bool sseActive = false;

void handleStream() {

WiFiClient client = server.client();

server.sendHeader("Cache-Control","no-cache");

server.sendHeader("Content-Type","text/event-stream");

server.sendHeader("Connection","keep-alive");

server.send(200, "text/event-stream", "");

sseClient = client;

sseActive = true;

}

void sseSend(const String &payload) {

if (!sseActive) return;

if (!sseClient.connected()) { sseActive = false; sseClient.stop(); return; }

sseClient.print("data: ");

sseClient.print(payload);

sseClient.print("\n\n");

sseClient.flush();

}

- handleStream() performs the initial SSE handshake (HTTP headers) and stores the client so the server can push updates later.

- sseSend() formats a message according to SSE protocol (data: <payload>\n\n) and sends it to the stored client.

20) HTTP handlers — how web UI & controls are implemented

- handleRoot() / handleStatic() / handleFavicon() serve static files from LittleFS.

- handleData() builds a StaticJsonDocument with engine, outer, gps, radar, crane, and alerts, serializes and sends it — this is simple snapshot API used by dashboard.

- handleControl() supports both GET (/control?servo=1&angle=90) and POST JSON for controlling servos:

  - servo=0 → radar immediate (writes servo directly)

  - servo=1/2 → set targetCrane1/2 so cranes move smoothly.

- handleServoRadar, handleServoCrane1, handleServoCrane2 are simple single-arg endpoints for convenience.

- handleApiCmd() accepts form or body-style requests for backward compatibility.

- handleDebug() returns current internal variables for debugging.

Important checks in handlers:

- They validate arguments (server.hasArg(...)) and return 400 (bad request) or 405 (method not allowed) when appropriate.

- constrain() ensures angles are within 0-180.

21) Setup() — initialization sequence

Key lines:

Serial.begin(115200);

pinMode(BUZZER_PIN, OUTPUT);

Wire.begin(SDA_PIN, SCL_PIN);

tcaselect(CH_OLED1); display1.init(); display1.flipScreenVertically();

tcaselect(CH_OLED2); display2.init(); display2.flipScreenVertically();

showBootAnimation();

dht_engine.begin(); dht_outer.begin();

analogReadResolution(12);

pinMode(US_TRIG_PIN, OUTPUT); pinMode(US_ECHO_PIN, INPUT);

servoRadar.attach(SERVO_RADAR_PIN);

GPS_Serial.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX);

tcaselect(CH_MPU); mpu.begin(); mpu.calcGyroOffsets(true);

LittleFS.begin(true);

WiFi.mode(WIFI_STA); WiFi.begin(WIFI_SSID, WIFI_PASS);

server.on(...); server.begin();

lastSensorMs = millis() - SENSOR_INTERVAL_MS;

- Initializes Serial for debugging.

- Initializes I2C and both OLEDs (remember to tcaselect before calling init()).

- Shows a boot animation to mark beginning of startup tasks.

- Initializes DHT sensors and sets ADC resolution to 12 bits to match 0–4095 range.

- Attaches servos and places them at initial positions.

- Starts GPS serial at 9600 baud.

- Initializes MPU and calibrates gyro offsets (blocking operation) — note: calibration expects the device to be still.

- Mounts LittleFS; starts Wi-Fi and webserver, registers routes.

- lastX = millis() - interval pattern means the first read/update happens immediately in the first loop iteration.

22) loop() — scheduling and the runtime behavior

void loop() {

unsigned long now = millis();

server.handleClient();

if (now - lastGpsMs >= GPS_INTERVAL_MS) { lastGpsMs = now; readGPS(); }

if (now - lastSensorMs >= SENSOR_INTERVAL_MS) { lastSensorMs = now; readSensors(); }

sweepRadar();

smoothMoveServo(...); // two cranes

if (now - lastOledMs >= OLED_INTERVAL_MS) { lastOledMs = now; updateDisplays(); }

// SSE periodic push

if (sseActive && sseClient.connected()) {

if (now - lastStreamMs >= STREAM_INTERVAL_MS) { ... build doc ...; sseSend(out); }

} else { if (sseActive && !sseClient.connected()) { sseActive=false; sseClient.stop(); } }

delay(1);

}

- server.handleClient() — processes incoming HTTP requests.

- The rest are non-blocking checks based on millis() to run tasks on schedules:

  - GPS (fast)

  - Sensors (every 1s)

  - Radar sweep is time-driven by its own lastRadarMs inside sweepRadar().

  - Smooth crane movement is called every loop — it moves a few degrees each time if needed.

  - OLED updates on its interval.

  - SSE pushes a JSON snapshot at STREAM_INTERVAL_MS (500ms).

- delay(1) yields briefly to prevent locking up and allows background tasks.